1 Identification

*

- Product Identifier

- Trade Name: Pure Clear plus Zero Phos.

- Relevant identified uses of the substance or mixture and uses advised against:

- Product Description: Pool water additive that aids in removal of organics and phosphates.

# Details of the Supplier of the Safety Data Sheet:

- Manufacturer/Supplier:

ReThink Poolcare Po Box 271722

Flower Mound, Tx, 75028 P: 800-592-0619

rethinkpoolcare.com care@rethinkpoolcare.com

- Emergency telephone number: 800-592-0619

2 Hazard(s) Identification

# Classification of the substance or mixture:

Corrosion

Eye damage 1	H318 Causes serious eye damage.

Skin irritation 2	H315 Causes skin irritation.

Sensitization - skin 1 H317 May cause an allergic skin reaction.

# Label elements:

- Hazard pictograms:

- Signal word: Danger

# Hazard-determining components of labeling:

lanthanum chloride, hydrate Alcohol C9-11

# Hazard statements:

H315 Causes skin irritation.

H318 Causes serious eye damage.

H317 May cause an allergic skin reaction.

# Precautionary statements:

P261	Avoid breathing dust/fume/gas/mist/vapors/spray.

P264	Wash thoroughly after handling.

P272	Contaminated work clothing must not be allowed out of the workplace. P280	Wear protective gloves / eye protection / face protection.

P302+P352	If on skin: Wash with plenty of water.

P305+P351+P338 If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P310	Immediately call a poison center/doctor.

(Contd. on page 2)

P321	 Specific treatment (see supplementary first aid instructions on this Safety Data Sheet). P362+P364	Take off contaminated clothing and wash it before reuse.

P332+P313	 If skin irritation occurs: Get medical advice/attention. P333+P313	If skin irritation or rash occurs: Get medical advice/attention.

P501	Dispose of contents/container in accordance with local/regional/national/international regulations.

# Unknown acute toxicity:

This value refers to knowledge of known, established toxicological or ecotoxicological values.

3.7 % of the mixture consists of component(s) of unknown toxicity.

# Information pertaining to particular dangers for man and environment:

- Classification system: NFPA/HMIS Definitions: 0-Least, 1-Slight, 2-Moderate, 3-High, 4-Extreme

# NFPA ratings (scale 0 - 4)

0

3	0

Health = 3

Fire = 0

Reactivity = 0

# HMIS-ratings (scale 0 - 4)

REACTIVITY 0

0

FIRE

HEALTH	*3

Health = *3 Fire = 0

Physical Hazard = 0

# Hazard(s) not otherwise classified (HNOC): None known

- Classification according to (d)(1)(ii) of § 1910.1200

The SDS issuer does not object to the classifications provided by importers or manufacturers of precursor products.

# Hazards not otherwise classified

There are no adverse physical or health effects known that are not covered by the hazard classes of the Hazard Communications Standard.

3 Composition/Information on Ingredients

*

# Chemical characterization: Substance

- Description:

Mixture of substances listed with non-hazardous additions and/or components that are less than listing threshold. All non-hazardous, components listed under threshold, and listed component total 100%.

| · Dangerous Components: |  |  |
| --- | --- | --- |
| 20211-76-1 | lanthanum chloride, hydrate | 2-12% |
|  | Corrosive to metals 1, H290; Skin corrosion 1B, H314; Eye damage 1, H318; Sensitization - skin 1B, H317 |  |
| 68439-46-3 | Alcohol C9-11 | 2-12% |
|  | Eye damage 1, H318; Skin irritation 2, H315 |  |
| 7757-82-6 | Sodium Sulphate | ≤2.5% |

# Additional information:

The exact percentages of the ingredients of this mixture are considered to be proprietary and are withheld in accordance with the provisions of paragraph (i) of §1910.1200 of 29 CFR 1910.1200 Trade Secrets.

4 First-Aid Measures

*

# Description of first aid measures

- General information: If symptoms persist, call a physician.

(Contd. on page 3)

# After inhalation:

In case of loss of consciousness, place patient securely in side position for transportaion. Remove to fresh air.

# After skin contact:

Wash with soap and water.

If skin irritation occurs, consult a doctor.

# After eye contact:

Rinse opened eye for several minutes under running water. If easy to do so, remove contact lenses if worn.

If eye irritation occurs, consult a doctor.

# After swallowing:

Rinse mouth with water.

If swallowed and symptoms occur, consult a doctor.

- Most important symptoms and effects, both acute and delayed: No further relevant information available.

- Indication of any immediate medical attention and special treatment needed: Treat symptomatically.

5 Fire-Fighting Measures

# Extinguishing media

- Suitable extinguishing agents: Use fire fighting measures that suit the environment.

- For safety reasons unsuitable extinguishing agents: No further relevant information.

- Special hazards arising from the substance or mixture: No further relevant information available.

# Advice for firefighters

- Special protective equipment for firefighters:

As in any fire, wear self-contained breathing apparatus pressure-demand (NIOSH approved or equivalent) and full protective gear to prevent contact with skin and eyes.

6 Accidental Release Measures

*

# Personal precautions, protective equipment and emergency procedures:

Avoid contact with skin, eyes and clothing. Avoid splashing.

- Environmental precautions: Dilute with plenty of water.

# Methods and material for containment and cleaning up:

Absorb with liquid-binding material (i.e. sand, diatomite, acid binders, universal binders, sawdust). Dispose of the collected material according to regulations.

| · PAC-1: |  |  |
| --- | --- | --- |
| 7757-82-6 | Sodium Sulphate | 9.8 mg/m³ |
| 9002-92-0 | Ethoxylated lauryl alkohol | 3.0 mg/m3 |
| 5989-27-5 | d-limonene | 15 ppm |
| 25155-30-0 | Sodium dodecylbenzenesulphonate | 2.1 mg/m³ |
| · PAC-2: |  |  |
| 7757-82-6 | Sodium Sulphate | 110 mg/m³ |
| 9002-92-0 | Ethoxylated lauryl alkohol | 33 mg/m3 |
| 5989-27-5 | d-limonene | 67 ppm |
| 25155-30-0 | Sodium dodecylbenzenesulphonate | 23 mg/m³ |
| · PAC-3: |  |  |
| 7757-82-6 | Sodium Sulphate | 650 mg/m³ |
| 9002-92-0 | Ethoxylated lauryl alkohol | 200 mg/m³ |
| 5989-27-5 | d-limonene | 170 ppm |

(Contd. on page 4)

| 25155-30-0 | Sodium dodecylbenzenesulphonate | 87 mg/m³ |
| --- | --- | --- |

# Reference to other sections:

See Section 7 for information on safe handling.

See Section 8 for information on personal protection equipment. See Section 13 for disposal information.

7 Handling and Storage

*

# Precautions for safe handling:

Avoid contact with skin, eyes and clothing Use gloves if handling.

- Information about protection against explosions and fires: No special measures required.

- Conditions for safe storage, including any incompatibilities

- Storage

- Requirements to be met by storerooms and receptacles:

Store at room temperature.

Avoid contact with strong acids and bases.

# Information about storage in one common storage facility: Not required.

- Further information about storage conditions: None.

- Specific end use(s): No further relevant information available.

8 Exposure Controls/Personal Protection

*

# Control parameters:

- Components with occupational exposure limits:

The following constituent is the only constituent of the product which has a PEL, TLV or other recommended exposure limit.

At this time, the other constituents have no known exposure limits.

| 7757-82-6 Sodium Sulphate |  |
| --- | --- |
| TLV | Short-term value: NIC-0.2 mg/m³ thoracic fraction of aerosol |

- Additional information: The lists that were valid during the creation of this SDS were used as basis.

# Exposure controls:

- Appropriate engineering controls No further data; see section 7.

# Personal protective equipment

- General protective and hygienic measures:

The usual precautionary measures for handling chemicals should be followed. Wash hands before breaks and at the end of work.

- Breathing equipment: Not required.

# Protection of hands:

Protective gloves

The glove material has to be impermeable and resistant to the product/ the substance/ the preparation.

# Material of gloves:

The selection of the suitable gloves does not only depend on the material, but also on further marks of quality and varies from manufacturer to manufacturer. As the product is a preparation of several substances, the resistance of the glove material cannot be calculated in advance and has therefore to be checked prior to the application.

(Contd. on page 5)

# Penetration time of glove material:

The exact break-through time has to be determined and observed by the manufacturer of the protective gloves.

# Eye protection:

Goggles recommended during refilling.

# Limitation and supervision of exposure into the environment: None

9 Physical and Chemical Properties

*

# Information on basic physical and chemical properties

- General Information

- Physical state	Liquid

- Color:	Light brown

- Odor:	Characteristic

- Odor threshold:	Not determined.

- Melting point/Melting range:	Not determined.

- Boiling point/Boiling range:	Not determined.

- Flammability:	Not applicable.

# Explosion limits:

- Lower:	Not determined.

- Upper:	Not determined.

- Flash point:	None

- Auto igniting:	Not applicable

- Decomposition temperature:	Not determined.

- pH-value:	Not determined.

# Viscosity:

- Kinematic:	Not determined.

- Dynamic:	Not determined.

# Solubility in / Miscibility with:

- Water:	Fully miscible.

- Partition coefficient (n-octanol/water):	Not determined.

- Vapor pressure @ 20 °C (68 °F):	≤23 hPa (≤17.3 mm Hg)

# Vapor pressure:

- Density:	Not determined.

- Relative density:	Not determined.

- Vapor density:	Not determined.

- Particle characteristics	Not applicable.

# Other information:

- Appearance:

- Form:	Liquid

# Important information on protection of health and environment, and on safety.

- Ignition temperature:	Product is not self-igniting.

- Danger of explosion:	Product does not present an explosion hazard.

# Change in condition

- Evaporation rate:	Not determined.

(Contd. on page 6)

* 10 Stability and Reactivity

- Reactivity: No further relevant information available.

- Chemical stability: Product is stable under normal conditions.

- Thermal decomposition / conditions to be avoided: No decomposition if used according to specifications.

- Possibility of hazardous reactions: No dangerous reactions known.

- Conditions to avoid: Heat and flame

- Incompatible materials: Strong acids and bases.

- Hazardous decomposition products: No dangerous decomposition products known.

* 11 Toxicological Information

# Information on toxicological effects:

- Acute toxicity:

- LD/LC50 values that are relevant for classification: No data available.

# Primary irritant effect:

- On the skin:

Strong caustic effect on skin and mucous membranes. May cause an allergic skin reaction.

# On the eye:

Strong irritant with the danger of severe eye injury. Corrosive effect.

Causes serious eye irritation.

# Additional toxicological information:

- Interactive effects No interactive effects between components are known.

# Carcinogenic categories:

| · IARC (International Agency for Research on Cancer): |  |  |
| --- | --- | --- |
| 5989-27-5 | d-limonene | 3 |
| · NTP (National Toxicology Program): |  |  |
| None of the ingredients are listed. |  |  |
| · OSHA-Ca (Occupational Safety & Health Administration): |  |  |
| None of the ingredients are listed. |  |  |

- Alternative sources for toxicological information

No non-standard sources for toxicological information where used.

* 12 Ecological Information

# Toxicity:

| · Aquatic toxicity: |  |
| --- | --- |
| 7757-82-6 Sodium Sulphate |  |
| EC50 | 2,564 mg/l (Water flea) |

- Persistence and degradability: No further relevant information available.

- Bioaccumulative potential: No further relevant information available.

- Mobility in soil: No further relevant information available.

# Results of PBT and vPvB assessment:

- PBT: Not applicable.

- vPvB: Not applicable.

(Contd. on page 7)

# Other adverse effects:

- Additional ecological information:

- General notes: Not known to be hazardous to water.

13 Disposal Considerations

# Waste treatment methods

- Recommendation:

Observe all federal, state and local environmental regulations when disposing of this material.

# Uncleaned packaging

- Recommendation: Disposal must be made according to official regulations.

- Recommended cleansing agent: Water, if necessary with cleansing agents.

* 14 Transport Information

# UN-Number:

- DOT, ADR/ADN, IMDG, IATA	Non-Regulated Material

# UN proper shipping name:

- DOT, ADR/ADN, IMDG, IATA	Non-Regulated Material

# Transport hazard class(es):

- DOT, ADR/ADN, ADN, IMDG, IATA

- Class:	Non-Regulated Material

# Packing group:

- DOT, ADR/ADN, IMDG, IATA	Non-Regulated Material

- Environmental hazards:	Not applicable.

# Transport in bulk according to Annex II of MARPOL73/78 and the IBC Code:	Not applicable.

- Special precautions for user:	Not applicable.

- UN "Model Regulation":	Non-Regulated Material

* 15 Regulatory Information

# Safety, health and environmental regulations/legislation specific for the substance or mixture:

No further relevant information available.

# SARA (Superfund Amendments and Reauthorization):

| · Section 355 (extremely hazardous substances): |  |  |
| --- | --- | --- |
| None of the ingredients are listed. |  |  |
| · Section 313 (Specific toxic chemical listings): |  |  |
| None of the ingredients are listed. |  |  |
| · TSCA (Toxic Substances Control Act): |  |  |
| 7732-18-5 | Water, distilled water, deionized water | ACTIVE |
| 68439-46-3 | Alcohol C9-11 | ACTIVE |
| 7757-82-6 | Sodium Sulphate | ACTIVE |
| 9002-92-0 | Ethoxylated lauryl alkohol | ACTIVE |
| 1300-72-7 | Sodium Xylene Sulfonate | ACTIVE |
| 77-92-9 | citric acid | ACTIVE |
| 5989-27-5 | d-limonene | ACTIVE |
| 25155-30-0 | Sodium dodecylbenzenesulphonate | ACTIVE |
| 61789-40-0 | Cocamidopropyl Betaine | ACTIVE |

(Contd. on page 8)

| 68584-25-8 | Benzenesulfonic acid, C10-16-alkyl derivs., compds. with triethanolamine | ACTIVE |
| --- | --- | --- |
| · Hazardous Air Pollutants |  |  |
| None of the ingredients are listed. |  |  |

- California Proposition 65:

| · Chemicals known to cause cancer: |  |  |  |
| --- | --- | --- | --- |
| None of the ingredients are listed. |  |  |  |
| · Chemicals known to cause reproductive toxicity for females: |  |  |  |
| None of the ingredients are listed. |  |  |  |
| · Chemicals known to cause reproductive toxicity for males: |  |  |  |
| None of the ingredients are listed. |  |  |  |
| · Chemicals known to cause developmental toxicity: |  |  |  |
| None of the ingredients are listed. |  |  |  |
| · New Jersey Right-to-Know List: |  |  |  |
| 25155-30-0 |  | Sodium dodecylbenzenesulphonate |  |
| · New Jersey Special Hazardous Substance List: |  |  |  |
| None of the ingredients are listed. |  |  |  |
| · Pennsylvania Right-to-Know List: |  |  |  |
| 7757-82-6 | Sodium Sulphate |  |  |
| · Pennsylvania Special Hazardous Substance List: |  |  |  |
| 7757-82-6 | Sodium Sulphate |  | E |

- Carcinogenic categories:

· EPA (Environmental Protection Agency):

None of the ingredients are listed.

· TLV (Threshold Limit Value established by ACGIH):

None of the ingredients are listed.

· NIOSH-Ca (National Institute for Occupational Safety and Health):

None of the ingredients are listed.

- GHS label elements

The product is classified and labeled according to the Globally Harmonized System (GHS).

# Hazard pictograms:

- Signal word: Danger

# Hazard-determining components of labeling:

lanthanum chloride, hydrate Alcohol C9-11

# Hazard statements:

H315 Causes skin irritation.

H318 Causes serious eye damage.

H317 May cause an allergic skin reaction.

# Precautionary statements:

P261	Avoid breathing dust/fume/gas/mist/vapors/spray.

(Contd. on page 9)

P264	Wash thoroughly after handling.

P272	Contaminated work clothing must not be allowed out of the workplace. P280	Wear protective gloves / eye protection / face protection.

P302+P352	If on skin: Wash with plenty of water.

P305+P351+P338 If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P310	Immediately call a poison center/doctor.

P321	 Specific treatment (see supplementary first aid instructions on this Safety Data Sheet). P362+P364	Take off contaminated clothing and wash it before reuse.

P332+P313	 If skin irritation occurs: Get medical advice/attention. P333+P313	If skin irritation or rash occurs: Get medical advice/attention.

P501	Dispose of contents/container in accordance with local/regional/national/international regulations.

· National regulations:

The product is not subject to be labelled according with the prevailing version of the regulations on hazardous substances.

- Chemical safety assessment: A Chemical Safety Assessment has not been carried out.

16 Other Information

The information and recommendations in this safety data sheet are, to the best of our knowledge, accurate as of the date of issue. Nothing herein shall be deemed to create warranty, expressed or implied, and shall not establish a legally valid contractual relationship. It is the responsibility of the user to determine applicability of this information and the suitability of the material or product for any particular purpose.

- Date of previous version 04/07/2025

# Version number of previous version: 1

- Date of preparation 04/07/2025

# Abbreviations and acronyms:

ADR: The European Agreement concerning the International Carriage of Dangerous Goods by Road

ADN: The European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways IMDG: International Maritime Code for Dangerous Goods

DOT: US Department of Transportation IATA: International Air Transport Association

EINECS: European Inventory of Existing Commercial Chemical Substances ELINCS: European List of Notified Chemical Substances

CAS: Chemical Abstracts Service (division of the American Chemical Society) NFPA: National Fire Protection Association (USA)

HMIS: Hazardous Materials Identification System (USA) LC50: Lethal concentration, 50 percent

LD50: Lethal dose, 50 percent

PBT: Persistent, Bioaccumulative and Toxic vPvB: very Persistent and very Bioaccumulative

NIOSH: National Institute for Occupational Safety and Health OSHA: Occupational Safety & Health Administration

TLV: Threshold Limit Value

PEL: Permissible Exposure Limit REL: Recommended Exposure Limit

Corrosive to metals 1: Corrosive to metals – Category 1 Skin corrosion 1B: Skin corrosion/irritation – Category 1B Skin irritation 2: Skin corrosion/irritation – Category 2

Eye damage 1: Serious eye damage/eye irritation – Category 1 Sensitization - skin 1: Skin sensitisation – Category 1 Sensitization - skin 1B: Skin sensitisation – Category 1B

# * Data compared to the previous version altered.

SDS created by MSDS Authoring Services	www.msdsauthoring.com	+1-877-204-9106