SAFETY DATA SHEET SunSmart Stabilizer

| Item | Information |
| --- | --- |
| Product identifier / Trade name | SunSmart Stabilizer |
| SDS version | Version 4.0 (reviewed and formatted) |
| Date of issue | May 06, 2026 |
| Revision date | May 06, 2026 |
| Previous version/date | Version 3 dated 02/10/2026 |
| Recommended use | Pool water treatment and stabilization |
| Prepared for | ReThink Poolcare, PO Box 271722, Flower Mound, TX 75028, USA |

# 1. Identification

| Item | Information |
| --- | --- |
| Product identifier | SunSmart Stabilizer |
| Product type | Mixture; aqueous liquid pool water treatment/stabilizer |
| Relevant identified use | Pool water treatment and stabilization |
| Uses advised against | Use only as directed on the product label. Do not mix with incompatible chemicals. |
| Manufacturer/Supplier | ReThink Poolcare, PO Box 271722, Flower Mound, TX 75028, USA |
| Telephone | 800-592-0619 |
| Website / Email | rethinkpoolcare.com / care@rethinkpoolcare.com |
| Emergency telephone | 1-888-255-3924 |

# 2. Hazard(s) Identification

Classification of the substance or mixture: This mixture is not classified as hazardous according to OSHA Hazard Communication Standard 29 CFR 1910.1200 and GHS Revision 7, based on the supplied formulation and available component information.

| Item | Information |
| --- | --- |
| GHS classification | Not classified as hazardous |
| Label elements | No GHS label elements required |
| Hazard pictograms | None required |
| Signal word | None required |
| Hazard statements | No known significant hazards under normal conditions of use |
| Precautionary statements | Avoid contact with eyes. Wash hands thoroughly after handling. Keep out of reach of children. Avoid uncontrolled release to the environment. |
| Hazards not otherwise classified (HNOC) | None known |
| Unknown acute toxicity | The acute toxicity of the complete mixture has not been tested; assessment is based on available ingredient information and concentration. |

| Rating System | Health | Flammability/Fire | Reactivity/Physical Hazard |
| --- | --- | --- | --- |
| NFPA (0-4) | 0 | 0 | 0 |
| HMIS (0-4) | 0 | 0 | 0 |

# 3. Composition/Information on Ingredients

Chemical characterization: Mixture. Ingredients requiring disclosure are listed below. Remaining ingredients are not classified as hazardous or are present below applicable reporting thresholds. Exact percentages may be withheld as trade secret where permitted by 29 CFR 1910.1200(i).

| Component | CAS No. | Function | Approx. % w/w | Classification / SDS Note |
| --- | --- | --- | --- | --- |
| Proprietary inorganic stabilizer | Trade Secret | Stabilizer | <1% | May contribute mild eye irritation depending on pH |
| Proprietary oxidizing agent | Trade Secret | Buffer / oxidizing agent | <1% | Low concentration in final blend; avoid mist/aerosol and uncontrolled environmental release |
| Proprietary mineral dispersion | Trade Secret | Active ingredient | <1% | Not expected to present respirable dust hazard in liquid, non-aerosol formulation |
| Proprietary rheology modifier | Trade Secret | Thickening agent | <1% | Not classified |
| Proprietary color additive | Trade Secret | Dye | <1% | Not classified at stated concentration |
| Deionized water | 7732-18-5 | Diluent | Balance to 100 | Not classified |

# 4. First-Aid Measures

| Item | Information |
| --- | --- |
| General information | If symptoms persist or discomfort occurs, seek medical attention. Show this SDS to medical personnel. |
| Inhalation | Move affected person to fresh air. Seek medical attention if symptoms develop or persist. |
| Skin contact | Wash thoroughly with soap and water. Remove contaminated clothing. Seek medical attention if irritation develops or persists. |
| Eye contact | Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do. Continue rinsing. Seek medical attention if irritation persists. |
| Ingestion | Rinse mouth with water. Do not induce vomiting unless directed by medical personnel. Seek medical attention if symptoms occur. |
| Most important symptoms/effects | May cause mild temporary eye or skin irritation. |
| Immediate medical attention/special treatment | Treat symptomatically. |

# 5. Fire-Fighting Measures

| Item | Information |
| --- | --- |
| Suitable extinguishing media | Use extinguishing media appropriate for surrounding fire conditions. |
| Unsuitable extinguishing media | None known. |
| Specific hazards arising from mixture | Product is not flammable. Thermal decomposition may produce irritating or hazardous fumes, including chlorine oxides and sodium oxides. |
| Advice for firefighters | Use standard firefighting procedures appropriate for the surrounding fire. |
| Special protective equipment | Wear self-contained breathing apparatus (NIOSH approved or equivalent) and full protective clothing. |

# 6. Accidental Release Measures

| Item | Information |
| --- | --- |
| Personal precautions | Avoid unnecessary contact with spilled material. Use appropriate PPE as described in Section 8. Ensure adequate ventilation during cleanup. |
| Environmental precautions | Prevent large quantities from entering drains, surface waters, waterways, or sewage systems. Inform authorities in case of significant release. |
| Containment and cleanup | Absorb spill with inert liquid-binding material such as sand, diatomite, universal binder, or sawdust. Collect into suitable containers for disposal. Wash spill area with water after cleanup. |
| Disposal of collected material | Dispose of collected material in accordance with applicable local, regional, national, and international regulations. |
| Reference to other sections | See Section 7 for handling/storage, Section 8 for PPE, and Section 13 for disposal considerations. |

# 7. Handling and Storage

| Item | Information |
| --- | --- |
| Precautions for safe handling | Ensure adequate ventilation. Avoid generation of mist or aerosol. Avoid contact with eyes and prolonged skin contact. Wash hands thoroughly after handling. |
| Protection against fire/explosion | No special fire or explosion protection measures required under normal conditions of use. |
| Storage conditions | Store in a cool, dry, well-ventilated area. Keep container tightly closed when not in use. |
| Storage requirements | Protect from freezing, excessive heat, and direct sunlight. |
| Incompatible materials | Strong acids, reducing agents, and strong oxidizing agents. |
| Specific end use | Pool water treatment and stabilization. |

# 8. Exposure Controls/Personal Protection

| Component | CAS No. | Occupational Exposure Limit / Guidance |
| --- | --- | --- |
| Proprietary oxidizing compound | Trade Secret | No established OSHA PEL. Avoid inhalation of mist or aerosol; use adequate ventilation. |
| Proprietary mineral-based active ingredient | Trade Secret | OSHA PEL total dust: 15 mg/m3; ACGIH TLV: 10 mg/m3. Dust limits are not normally applicable to liquid, non-aerosol product use. |

| Item | Information |
| --- | --- |
| Engineering controls | Use adequate general ventilation during handling and use. Minimize exposure to mist or aerosol. |
| Respiratory protection | Not normally required under normal conditions of use with adequate ventilation. |
| Hand protection | Chemical-resistant gloves such as nitrile or neoprene are recommended during prolonged or repeated handling. |
| Eye protection | Safety glasses or chemical splash goggles are recommended to avoid eye contact. |
| Skin/body protection | Wear appropriate protective clothing as necessary to minimize skin contact. |
| Hygiene measures | Wash hands before breaks and at end of work. Keep away from foodstuffs, beverages, and animal feed. |
| Environmental exposure controls | Avoid uncontrolled release to the environment. Prevent large quantities from entering drains, waterways, or sewage systems. |

# 9. Physical and Chemical Properties

| Property | Value |
| --- | --- |
| Physical state | Liquid |
| Color | Clear to orange |
| Odor | Characteristic |
| Odor threshold | Not determined |
| pH | Not determined |
| Melting/freezing point | Not determined |
| Boiling point/range | Not determined |
| Flash point | Not applicable; aqueous non-flammable mixture |
| Flammability | Not applicable |
| Lower/upper explosive limits | Not determined / Not determined |
| Vapor pressure | Not determined |
| Vapor density | Not determined |
| Relative density | Not determined |
| Solubility in water | Miscible/dispersible in water |
| Partition coefficient n-octanol/water | Not determined |
| Auto-ignition temperature | Not applicable; product is not self-igniting |
| Decomposition temperature | Not determined |
| Viscosity | Not determined |
| Particle characteristics | Not applicable to liquid |
| VOC content | 0.00% |
| Evaporation rate | Not determined |
| Explosion hazard | Product does not present an explosion hazard under normal conditions |

# 10. Stability and Reactivity

| Item | Information |
| --- | --- |
| Reactivity | No dangerous reactivity expected under normal conditions of use and storage. |
| Chemical stability | Stable under normal conditions. |
| Possibility of hazardous reactions | No dangerous reactions known under recommended use conditions. |
| Conditions to avoid | Avoid freezing, excessive heat, direct sunlight, strong acids, reducing agents, and strong oxidizing agents. |
| Incompatible materials | Strong acids, reducing agents, and strong oxidizing agents. |
| Hazardous decomposition products | Thermal decomposition may produce chlorine oxides and sodium oxides. |

# 11. Toxicological Information

Toxicological information is based on available data for individual components and the mixture under normal conditions of use. The complete mixture has not been fully tested.

| Endpoint / Item | Information |
| --- | --- |
| Acute toxicity | Based on available information and stated concentrations, the mixture is not expected to present significant acute toxicity hazards under intended conditions of use. |
| Oral toxicity | Oral LD50 (rat): 284 mg/kg. |
| Dermal toxicity | Dermal LD50 (rabbit): 134 mg/kg. |
| Skin irritation | Prolonged or repeated exposure may cause mild skin irritation. Concentrated sodium chlorite solutions may cause irritation or corrosive effects; finished mixture concentration is low. |
| Eye irritation | May cause temporary eye irritation. Avoid direct eye contact. |
| Sensitization | Sodium chlorite: OECD TG 406 Guinea Pig Maximisation Test - not considered a skin sensitizer. |
| Interactive effects | No known significant interactive toxicological effects between components. |
| Carcinogenicity - IARC | Not classifiable as to carcinogenicity to humans and in inhalation not expected to present significant hazard in liquid formulation. |
| NTP / OSHA carcinogens | None of the ingredients are listed at relevant conditions of exposure. |

# 12. Ecological Information

| Endpoint / Item | Information |
| --- | --- |
| Ecotoxicity basis | Information is based on available data for individual components and the mixture under normal conditions of use. |
| Sodium chlorite - algae | EC50 green algae: 1 mg/L, 96 h (static, US-EPA). |
| Sodium chlorite - daphnia | EC50 Daphnia: <1 mg/L, 48 h (OECD TG 202). |
| Sodium chlorite - activated sludge | EC50 activated sludge: >100 mg/L, 3 h (OECD TG 209). |
| Persistence and degradability | No specific data available for the mixture. |
| Bioaccumulative potential | No specific data available for the mixture. |
| Mobility in soil | No specific data available for the mixture. |
| PBT/vPvB assessment | PBT: Not applicable. vPvB: Not applicable. |
| Other adverse effects | Large uncontrolled releases may be harmful to aquatic organisms. Avoid uncontrolled release to the environment. |

# 13. Disposal Considerations

| Item | Information |
| --- | --- |
| Waste treatment methods | Dispose of contents and container in accordance with applicable local, regional, national, and international regulations. |
| Disposal recommendation | Do not dispose of with household waste. Prevent product from entering drains, sewage systems, or natural waterways. |
| Contaminated packaging | Empty containers may retain product residues and should be handled appropriately. |
| Uncleaned packaging | Dispose of contaminated packaging in accordance with applicable official regulations. |
| Environmental precautions during disposal | Avoid uncontrolled release into the environment during disposal operations. |

# 14. Transport Information

| Item | Information |
| --- | --- |
| UN number | Not regulated as dangerous goods under DOT, ADR/ADN, IMDG, or IATA regulations. |
| UN proper shipping name | Not regulated as dangerous goods. |
| Transport hazard class(es) | Not regulated. |
| Packing group | Not applicable. |
| Environmental hazards | Not classified as environmentally hazardous for transport purposes. |
| Marine pollutant | No. |
| Transport in bulk according to MARPOL/IBC Code | Not applicable. |
| Special precautions for user | Transport in closed, properly labeled containers. Avoid freezing and excessive heat during transport. |
| DOT / ADR / ADN / IMDG / IATA | Not regulated. |

# 15. Regulatory Information

| Regulation / List | Status |
| --- | --- |
| OSHA HazCom / GHS | This mixture is not classified as hazardous according to OSHA Hazard Communication Standard 29 CFR 1910.1200 and GHS Revision 7. |
| SARA Section 355 - Extremely Hazardous Substances | None of the ingredients are listed. |
| SARA Section 313 - Specific Toxic Chemical Listings | None of the ingredients are listed at reportable concentrations. |
| TSCA | All intentionally added components are expected to comply with TSCA inventory requirements. |
| Hazardous Air Pollutants (HAPs) | None of the ingredients are listed. |
| California Proposition 65 | Trade Secret used is listed only for airborne, unbound particles of respirable size. In this liquid, non-aerosol product, respirable dust exposure is not expected under normal use. |
| New Jersey Right-to-Know | Trade Secret |
| Pennsylvania Right-to-Know | Trade Secret |
| ACGIH TLV classification | Trade Secret: A4 - Not classifiable as a human carcinogen. |
| NIOSH carcinogen list | None of the ingredients are listed at relevant conditions of exposure. |
| Chemical safety assessment | A chemical safety assessment has not been carried out for the mixture. |

# 16. Other Information

| Item | Information |
| --- | --- |
| Date of previous version | July 02, 2025 |
| Version number of previous version | 3 |
| Date of preparation | May 06, 2026 |
| Revision information | SDS reviewed, formatted, duplicate sections removed, internal date format standardized, and formulation/regulatory wording aligned. |
| Disclaimer | The information and recommendations in this SDS are believed to be accurate as of the date of issue and are based on information provided for the product. No warranty, expressed or implied, is made. It is the responsibility of the user to determine the applicability and suitability of this information for a particular purpose and to comply with all applicable laws and regulations. |

| Abbreviation | Meaning |
| --- | --- |
| ADR | European Agreement concerning the International Carriage of Dangerous Goods by Road |
| ADN | European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways |
| IMDG | International Maritime Dangerous Goods Code |
| DOT | United States Department of Transportation |
| IATA | International Air Transport Association |
| CAS | Chemical Abstracts Service |
| NFPA | National Fire Protection Association |
| HMIS | Hazardous Materials Identification System |
| VOC | Volatile Organic Compounds |
| LC50 | Lethal Concentration, 50% |
| LD50 | Lethal Dose, 50% |
| PBT | Persistent, Bioaccumulative and Toxic |
| vPvB | Very Persistent and Very Bioaccumulative |
| NIOSH | National Institute for Occupational Safety and Health |
| OSHA | Occupational Safety and Health Administration |
| TLV | Threshold Limit Value |
| PEL | Permissible Exposure Limit |
| REL | Recommended Exposure Limit |
| OECD | Organisation for Economic Co-operation and Development |
| TSCA | Toxic Substances Control Act |
| GHS | Globally Harmonized System of Classification and Labelling of Chemicals |

End of Safety Data Sheet